# ARM外围传感器实验 — 上午实验代码

实验七：CAN总线自发自收实验 | 实验八：LCD液晶屏实验 | 实验九：XPT触摸屏实验

# 实验七 CAN总线自发自收实验

【代码7-1-3】BSP/can.h — CAN总线对外接口头文件

📄 来源文件: can.h

#ifndef __CAN_H__

#define __CAN_H__

#include "stm32f4xx.h"

void CAN1_Init(void);

void CAN1_SendMsg(uint32_t uID, uint8_t *pData, uint32_t uLen);

#endif

【代码7-1-4】BSP/can.c — CAN总线驱动基础定义

📄 来源文件: can.c

#include <stdio.h>

#include <string.h>

#include "stm32f4xx.h"

void CAN1_FilterConfig(void);

【代码7-2-1】BSP/can.c — CAN初始化（GPIO+CAN控制器+中断）

📄 来源文件: can.c

void CAN1_Init(void) {

GPIO_InitTypeDef GPIO_InitStruct = {0};

CAN_InitTypeDef  CAN_InitStruct={0};

NVIC_InitTypeDef  NVIC_InitStructure;

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA, ENABLE);//使能GPIO时钟

RCC_APB1PeriphClockCmd(RCC_APB1Periph_CAN1, ENABLE);//使能串口时钟

//PA11、PA12复用为CAN1

GPIO_InitStruct.GPIO_Pin  = GPIO_Pin_11 | GPIO_Pin_12;

GPIO_InitStruct.GPIO_Mode = GPIO_Mode_AF;

GPIO_InitStruct.GPIO_OType = GPIO_OType_PP;

GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_NOPULL;

GPIO_InitStruct.GPIO_Speed = GPIO_Speed_100MHz;

GPIO_Init(GPIOA, &GPIO_InitStruct);

//引脚复用映射配置

GPIO_PinAFConfig(GPIOA, GPIO_PinSource11, GPIO_AF_CAN1);

GPIO_PinAFConfig(GPIOA, GPIO_PinSource12, GPIO_AF_CAN1);

CAN_InitStruct.CAN_Prescaler = 6;       //分频系数(Fdiv)为brp+1

CAN_InitStruct.CAN_Mode = CAN_Mode_LoopBack;     //自发自收模式

CAN_InitStruct.CAN_SJW  = CAN_SJW_1tq;    //重新同步跳跃宽度

CAN_InitStruct.CAN_BS1  = CAN_BS1_6tq;  //Tbs1范围

CAN_InitStruct.CAN_BS2  = CAN_BS2_7tq;  //Tbs2范围

CAN_InitStruct.CAN_TTCM = DISABLE;    //非时间触发通信模式

CAN_InitStruct.CAN_ABOM = DISABLE;    //软件自动离线管理

CAN_InitStruct.CAN_AWUM = DISABLE;  //睡眠模式通过软件唤醒

CAN_InitStruct.CAN_NART = ENABLE;    //禁止报文自动传送

CAN_InitStruct.CAN_RFLM = DISABLE;    //报文不锁定,新的覆盖旧的

CAN_InitStruct.CAN_TXFP = DISABLE;    //优先级由报文标识符决定

CAN_Init(CAN1, &CAN_InitStruct);     // 初始化CAN1

//中断配置

CAN_ITConfig(CAN1, CAN_IT_FMP0, ENABLE);//FIFO0消息挂号中断允许

NVIC_InitStructure.NVIC_IRQChannel = CAN1_RX0_IRQn;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 1;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 0;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_Init(&NVIC_InitStructure);

CAN1_FilterConfig();

}

【代码7-2-2】BSP/can.c — CAN筛选器配置

📄 来源文件: can.c

//接收筛选器配置

void CAN1_FilterConfig(void){

CAN_FilterInitTypeDef  CAN_Filter;

//配置过滤器

CAN_Filter.CAN_FilterNumber     = 0;      //过滤器0

CAN_Filter.CAN_FilterMode       = CAN_FilterMode_IdMask;

CAN_Filter.CAN_FilterScale      = CAN_FilterScale_32bit; //32位

CAN_Filter.CAN_FilterIdHigh     = 0x0000;//32位ID

CAN_Filter.CAN_FilterIdLow      = 0x0000;

CAN_Filter.CAN_FilterMaskIdHigh = 0x0000;//32位MASK

CAN_Filter.CAN_FilterMaskIdLow  = 0x0000;

CAN_Filter.CAN_FilterFIFOAssignment = CAN_Filter_FIFO0;//关联到FIFO0

CAN_Filter.CAN_FilterActivation     = ENABLE; //激活过滤器0

CAN_FilterInit(&CAN_Filter);//滤波器初始化

}

【代码7-2-3】BSP/can.c — CAN接收中断服务函数

📄 来源文件: can.c

//中断服务函数

void CAN1_RX0_IRQHandler(void){

int i = 0;

uint32_t uCanID = 0;

CanRxMsg RxMessage;

CAN_Receive(CAN1, 0, &RxMessage);//接收数据

uCanID = (RxMessage.IDE == CAN_Id_Standard) ? RxMessage.StdId : RxMessage.ExtId;

printf("\nRecv CANID:0x%08X Data:[", uCanID);

for(i=0; i<8; i++){

printf("%02X ", RxMessage.Data[i]);

}

printf("]\r\n");

}

【代码7-2-4】BSP/can.c — CAN发送消息函数

📄 来源文件: can.c

//发送消息

void CAN1_SendMsg(uint32_t uID, uint8_t *pData, uint32_t uLen){

uint32_t i =0;

uint8_t uMailBox = 0;

CanTxMsg TxMessage;

TxMessage.StdId = uID;     // 标准标识符

TxMessage.ExtId = uID;     // 扩展标示符（29位）

TxMessage.IDE   = CAN_Id_Extended;    // 使用扩展标识符

TxMessage.RTR   = CAN_RTR_Data;       // 数据帧

TxMessage.DLC   = uLen;              // 发送数据长度

printf("Send CAN Data:[0x%04X: ", uID);

for(i=0; i<uLen; i++){

printf("%02X ", pData[i]);

TxMessage.Data[i] = pData[i];    // 填充数据

}

printf("]\r\n");

uMailBox = CAN_Transmit(CAN1, &TxMessage);

i = 0;

while((CAN_TransmitStatus(CAN1, uMailBox)==CAN_TxStatus_Failed)) {

i++;

if(i>=0XFFF){ break; }

}

if(i>=0xFFF){

printf("Send Failed!\r\n");

return;

}

}

【代码7-2-5】USER/main.c — CAN总线测试主程序

📄 来源文件: main.c

#include "stm32f4xx.h"

#include "usart.h"

#include "delay.h"

#include "can.h"

int main(void){

uint8_t  pTxData[8] = {0x88, 0x77, 0x66, 0x55, 0x44, 0x33, 0x22, 0x11};

UART2_Init(115200);

CAN1_Init();

while( 1 ) {

CAN1_SendMsg(0x1234, pTxData, 8);

delay_ms(5000);

}

}

【代码7-1-2】BSP/usart.c — 调试串口UART2驱动（printf重定向+中断收发）

📄 来源文件: usart.c

#include <stdio.h>

#include "usart.h"

void UART2_Init(u32 bound)

{

GPIO_InitTypeDef GPIO_InitStructure;

USART_InitTypeDef USART_InitStructure;

NVIC_InitTypeDef NVIC_InitStructure;

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA,ENABLE);

RCC_APB1PeriphClockCmd(RCC_APB1Periph_USART2,ENABLE);

GPIO_PinAFConfig(GPIOA, GPIO_PinSource2, GPIO_AF_USART2);

GPIO_PinAFConfig(GPIOA, GPIO_PinSource3, GPIO_AF_USART2);

GPIO_InitStructure.GPIO_Pin  = GPIO_Pin_2 | GPIO_Pin_3;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIO_InitStructure.GPIO_PuPd  = GPIO_PuPd_UP;

GPIO_Init(GPIOA,&GPIO_InitStructure);

USART_InitStructure.USART_BaudRate = bound;

USART_InitStructure.USART_WordLength = USART_WordLength_8b;

USART_InitStructure.USART_StopBits   = USART_StopBits_1;

USART_InitStructure.USART_Parity     = USART_Parity_No;

USART_InitStructure.USART_HardwareFlowControl = USART_HardwareFlowControl_None;

USART_InitStructure.USART_Mode = USART_Mode_Rx | USART_Mode_Tx;

USART_Init(USART2, &USART_InitStructure);

NVIC_InitStructure.NVIC_IRQChannel = USART2_IRQn;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 3;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 3;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_Init( &NVIC_InitStructure );

USART_ITConfig(USART2, USART_IT_RXNE, ENABLE);

USART_Cmd(USART2, ENABLE);

}

void UART_SendByte( USART_TypeDef * pUSARTx, uint8_t ch)

{

USART_SendData(pUSARTx, ch);

while (USART_GetFlagStatus(pUSARTx, USART_FLAG_TXE) == RESET);

}

void UART_SendString( USART_TypeDef * pUSARTx, char *str)

{

unsigned int k=0;

do {

UART_SendByte( pUSARTx, *(str + k) );

k++;

} while(*(str + k)!='\0');

while(USART_GetFlagStatus(pUSARTx,USART_FLAG_TC)==RESET){}

}

void UART_SendHalfWord( USART_TypeDef * pUSARTx, uint16_t ch)

{

uint8_t temp_h, temp_l;

temp_h = (ch&0XFF00)>>8;

temp_l = ch&0XFF;

USART_SendData(pUSARTx,temp_h);

while (USART_GetFlagStatus(pUSARTx, USART_FLAG_TXE) == RESET);

USART_SendData(pUSARTx,temp_l);

while (USART_GetFlagStatus(pUSARTx, USART_FLAG_TXE) == RESET);

}

int fputc(int ch, FILE *f)

{

USART_SendData(USART2, (uint8_t) ch);

while (USART_GetFlagStatus(USART2, USART_FLAG_TXE) == RESET);

return (ch);

}

int fgetc(FILE *f){

while (USART_GetFlagStatus(USART2, USART_FLAG_RXNE) == RESET);

return (int)USART_ReceiveData(USART2);

}

void USART2_IRQHandler( void ){

u8 sbuf = 0;

if(USART_GetFlagStatus(USART2, USART_FLAG_RXNE)) {

sbuf = USART_ReceiveData(USART2);

USART_SendData(USART2, sbuf);

}

}

【代码7-1-1】BSP/usart.h — 调试串口接口头文件

📄 来源文件: usart.h

#ifndef __USART_H__

#define __USART_H__

#include "stm32f4xx.h"

void UART2_Init(u32 bound);

#endif

# 实验八 LCD液晶屏实验

【代码7-1-1】BSP/lcd_ili9341.h — LCD对外接口头文件（颜色宏+API声明）

📄 来源文件: lcd_ili9341.h

#ifndef __LCD_ILI9341_H__

#define    __LCD_ILI9341_H__

#include "stm32f4xx.h"

#include "lcd_fonts.h"

#define      WHITE                           0xFFFF

#define      BLACK                         0x0000

#define      GREY                          0xF7DE

#define      BLUE                          0x001F

#define      BLUE2                         0x051F

#define      RED                           0xF800

#define      MAGENTA                       0xF81F

#define      GREEN                         0x07E0

#define      CYAN                          0x7FFF

#define      YELLOW                        0xFFE0

#define      BRED                          0xF81F

#define      GRED                          0xFFE0

#define      GBLUE                         0x07FF

void LCD_Init(void);

uint16_t LCD_GetLenX(void);

uint16_t LCD_GetLenY(void);

void LCD_SetColors(uint16_t TextColor, uint16_t BackColor);

void LCD_GetColors(uint16_t *TextColor, uint16_t *BackColor);

void LCD_SetTextColor(uint16_t Color);

void LCD_SetBackColor(uint16_t Color);

void LCD_SetPixel(uint16_t usX, uint16_t usY);

uint16_t LCD_GetPixel(uint16_t usX, uint16_t usY);

void LCD_Clear(uint16_t usX, uint16_t usY, uint16_t usWidth, uint16_t usHeight);

void LCD_DrawLine(uint16_t usX1, uint16_t usY1, uint16_t usX2, uint16_t usY2);

void LCD_DrawRectangle(uint16_t usX_Start, uint16_t usY_Start,

uint16_t usWidth, uint16_t usHeight, uint8_t ucFilled);

void LCD_DrawCircle(uint16_t usX_Center, uint16_t usY_Center,

uint16_t usRadius, uint8_t ucFilled);

void LCD_DispStringEN(uint16_t usX, uint16_t usY, uint8_t uDir, char *pStr);

#endif

【代码7-1-2】BSP/lcd_ili9341.c — LCD驱动全局定义与FSMC地址宏

📄 来源文件: lcd_ili9341.c

#include <stddef.h>

#include "lcd_ili9341.h"

#include "lcd_fonts.h"

#include "delay.h"

//FSMC_Bank1_NORSRAM用于LCD命令操作的地址

#define FSMC_Addr_ILI9341_CMD   ((uint32_t)0x60000000)

//FSMC_Bank1_NORSRAM用于LCD数据操作的地址

#define FSMC_Addr_ILI9341_DATA  ((uint32_t)0x60020000)

//复位引脚

#define  ILI9341_RST_PORT    GPIOE

#define  ILI9341_RST_PIN     GPIO_Pin_1

//背光引脚

#define  ILI9341_BK_PORT     GPIOD

#define  ILI9341_BK_PIN      GPIO_Pin_12

#define   ILI9341_LESS_PIXEL     240

#define   ILI9341_MORE_PIXEL     320

//ILI9341命令定义

#define CMD_RESET         0x0001

#define CMD_SLEEP_OUT    0x0011

#define CMD_DISPLAY_OFF    0x0028

#define CMD_DISPLAY_ON    0x0029

#define CMD_COLUMN_ADDR    0x002A

#define CMD_PAGE_ADDR    0x002B

#define CMD_GRAM        0x002C

#define CMD_MAC            0x0036

#define CMD_PIXEL_FORMAT   0x003A

//向ILI9341写入命令

#define ILI9341_WriteCmd(usCmd)  *(__IO uint16_t*)(FSMC_Addr_ILI9341_CMD)=usCmd;

//向ILI9341写入数据

#define ILI9341_WriteData(usData) *(__IO uint16_t *)(FSMC_Addr_ILI9341_DATA)=usData;

//从ILI9341读取数据

#define ILI9341_ReadData()   (*(__IO uint16_t *)(FSMC_Addr_ILI9341_DATA));

uint8_t g_LCD_ScanMode = 2;

static uint16_t g_LCD_TextColor   = BLACK;

static uint16_t g_LCD_BackColor   = WHITE;

uint16_t g_LCD_LenX = ILI9341_LESS_PIXEL;

uint16_t g_LCD_LenY = ILI9341_MORE_PIXEL;

static void ILI9341_GPIO_Config( void );

static void ILI9341_FSMC_Config( void );

static void ILI9341_Rst( void );

static void ILI9341_REG_Config( void );

static void ILI9341_FillColor(uint32_t ulAmout_Point, uint16_t usColor);

static void ILI9341_BackLight(uint8_t uOnOff);

static void ILI9341_GramScan(uint8_t ucOption);

static void ILI9341_OpenWindow(uint16_t usX, uint16_t usY,

uint16_t usWidth, uint16_t usHeight);

uint16_t LCD_GetLenX(void){ return g_LCD_LenX; }

uint16_t LCD_GetLenY(void){ return g_LCD_LenY; }

【代码7-2-1】BSP/lcd_ili9341.c — GPIO初始化（FSMC数据/控制引脚复用）

📄 来源文件: lcd_ili9341.c

static void ILI9341_GPIO_Config ( void ){

GPIO_InitTypeDef GPIO_InitStructure;

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOD | RCC_AHB1Periph_GPIOE, ENABLE);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_0 | GPIO_Pin_1 | GPIO_Pin_4 |

GPIO_Pin_5 | GPIO_Pin_7 | GPIO_Pin_8 | GPIO_Pin_9 |

GPIO_Pin_10 | GPIO_Pin_11 | GPIO_Pin_14 | GPIO_Pin_15;

GPIO_InitStructure.GPIO_Mode  = GPIO_Mode_AF;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIO_InitStructure.GPIO_PuPd  = GPIO_PuPd_NOPULL;

GPIO_Init(GPIOD, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_7 | GPIO_Pin_8 | GPIO_Pin_9 |

GPIO_Pin_10 | GPIO_Pin_11 | GPIO_Pin_12 | GPIO_Pin_13 |

GPIO_Pin_14 | GPIO_Pin_15;

GPIO_Init(GPIOE, &GPIO_InitStructure);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource0, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource1, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource4, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource5, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource7, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource8, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource9, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource10, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource11, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource14, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOD, GPIO_PinSource15, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource7, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource8, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource9, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource10, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource11, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource12, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource13, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource14, GPIO_AF_FSMC);

GPIO_PinAFConfig(GPIOE, GPIO_PinSource15, GPIO_AF_FSMC);

//LED背光

GPIO_InitStructure.GPIO_Pin   = GPIO_Pin_12;

GPIO_InitStructure.GPIO_Mode  = GPIO_Mode_OUT;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_2MHz;

GPIO_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIO_InitStructure.GPIO_PuPd  = GPIO_PuPd_NOPULL;

GPIO_Init(GPIOD, &GPIO_InitStructure);

//RESET

GPIO_InitStructure.GPIO_Pin   = GPIO_Pin_1;

GPIO_Init(GPIOE, &GPIO_InitStructure);

}

【代码7-2-2】BSP/lcd_ili9341.c — 背光控制与软件复位

📄 来源文件: lcd_ili9341.c

void ILI9341_BackLight(uint8_t uOnOff){

if(uOnOff){

GPIO_ResetBits(ILI9341_BK_PORT, ILI9341_BK_PIN);

} else {

GPIO_SetBits(ILI9341_BK_PORT, ILI9341_BK_PIN);

}

}

void ILI9341_Rst(void){

GPIO_ResetBits(ILI9341_RST_PORT, ILI9341_RST_PIN);

ILI9341_DELAY_MS(5);

GPIO_SetBits(ILI9341_RST_PORT, ILI9341_RST_PIN);

ILI9341_DELAY_MS(5);

}

【代码7-2-3】BSP/lcd_ili9341.c — FSMC接口初始化（SRAM Bank1模拟8080）

📄 来源文件: lcd_ili9341.c

static void ILI9341_FSMC_Config(void){

FSMC_NORSRAMInitTypeDef  FSMC_NORSRAMInitStructure;

FSMC_NORSRAMTimingInitTypeDef  p;

RCC_AHB3PeriphClockCmd(RCC_AHB3Periph_FSMC, ENABLE);

p.FSMC_AddressSetupTime = 0x6;

p.FSMC_AddressHoldTime  = 0;

p.FSMC_DataSetupTime    = 0x6;

p.FSMC_BusTurnAroundDuration = 0;

p.FSMC_CLKDivision = 0;

p.FSMC_DataLatency = 0;

p.FSMC_AccessMode = FSMC_AccessMode_A;

FSMC_NORSRAMInitStructure.FSMC_Bank = FSMC_Bank1_NORSRAM1;

FSMC_NORSRAMInitStructure.FSMC_DataAddressMux = FSMC_DataAddressMux_Disable;

FSMC_NORSRAMInitStructure.FSMC_MemoryType = FSMC_MemoryType_SRAM;

FSMC_NORSRAMInitStructure.FSMC_MemoryDataWidth = FSMC_MemoryDataWidth_16b;

FSMC_NORSRAMInitStructure.FSMC_BurstAccessMode = FSMC_BurstAccessMode_Disable;

FSMC_NORSRAMInitStructure.FSMC_AsynchronousWait = FSMC_AsynchronousWait_Disable;

FSMC_NORSRAMInitStructure.FSMC_WaitSignalPolarity = FSMC_WaitSignalPolarity_Low;

FSMC_NORSRAMInitStructure.FSMC_WrapMode = FSMC_WrapMode_Disable;

FSMC_NORSRAMInitStructure.FSMC_WaitSignalActive = FSMC_WaitSignalActive_BeforeWaitState;

FSMC_NORSRAMInitStructure.FSMC_WriteOperation = FSMC_WriteOperation_Enable;

FSMC_NORSRAMInitStructure.FSMC_WaitSignal = FSMC_WaitSignal_Disable;

FSMC_NORSRAMInitStructure.FSMC_ExtendedMode = FSMC_ExtendedMode_Disable;

FSMC_NORSRAMInitStructure.FSMC_WriteBurst = FSMC_WriteBurst_Disable;

FSMC_NORSRAMInitStructure.FSMC_ReadWriteTimingStruct = &p;

FSMC_NORSRAMInitStructure.FSMC_WriteTimingStruct = &p;

FSMC_NORSRAMInit(&FSMC_NORSRAMInitStructure);

FSMC_NORSRAMCmd(FSMC_Bank1_NORSRAM1, ENABLE);

}

【代码7-2-5】BSP/lcd_ili9341.c — ILI9341寄存器初始化配置

📄 来源文件: lcd_ili9341.c

static void ILI9341_REG_Config(void){

ILI9341_WriteCmd(CMD_RESET);

ILI9341_DELAY_MS(120);

ILI9341_WriteCmd(CMD_DISPLAY_OFF);

ILI9341_WriteCmd(CMD_PIXEL_FORMAT);

ILI9341_WriteData(0x55); // 16bit/pixel

ILI9341_WriteCmd(CMD_SLEEP_OUT);

ILI9341_DELAY_MS(100);

ILI9341_WriteCmd(CMD_DISPLAY_ON);

ILI9341_DELAY_MS(100);

ILI9341_WriteCmd(CMD_GRAM);

ILI9341_DELAY_MS(5);

}

【代码7-2-6】BSP/lcd_ili9341.c — LCD总初始化接口

📄 来源文件: lcd_ili9341.c

void LCD_Init(void){

ILI9341_GPIO_Config();

ILI9341_FSMC_Config();

ILI9341_Rst();

ILI9341_BackLight(ENABLE);

ILI9341_REG_Config();

ILI9341_GramScan(g_LCD_ScanMode);

}

【代码7-3-1】BSP/lcd_ili9341.c — 显示窗口申请

📄 来源文件: lcd_ili9341.c

void ILI9341_OpenWindow(uint16_t usX, uint16_t usY,

uint16_t usWidth, uint16_t usHeight)

{

ILI9341_WriteCmd(CMD_SetCoordinateX);

ILI9341_WriteData(usX >> 8);

ILI9341_WriteData(usX & 0xff);

ILI9341_WriteData((usX + usWidth - 1) >> 8);

ILI9341_WriteData((usX + usWidth - 1) & 0xff);

ILI9341_WriteCmd(CMD_SetCoordinateY);

ILI9341_WriteData(usY >> 8);

ILI9341_WriteData(usY & 0xff);

ILI9341_WriteData((usY + usHeight - 1) >> 8);

ILI9341_WriteData((usY + usHeight - 1) & 0xff);

}

【代码7-3-2】BSP/lcd_ili9341.c — LCD扫描模式设置

📄 来源文件: lcd_ili9341.c

void ILI9341_GramScan(uint8_t ucOption){

if(ucOption > 7){ return; }

g_LCD_ScanMode = ucOption;

if(ucOption%2 == 0){

g_LCD_LenX = ILI9341_LESS_PIXEL;

g_LCD_LenY = ILI9341_MORE_PIXEL;

} else {

g_LCD_LenX = ILI9341_MORE_PIXEL;

g_LCD_LenY = ILI9341_LESS_PIXEL;

}

ILI9341_WriteCmd(CMD_MAC);

ILI9341_WriteData(0x08 | (ucOption<<5));

ILI9341_OpenWindow(0, 0, g_LCD_LenX, g_LCD_LenY);

ILI9341_WriteCmd(CMD_SetPixel);

}

【代码7-3-3~5】BSP/lcd_ili9341.c — 颜色填充、前景/背景色、像素设置、清屏

📄 来源文件: lcd_ili9341.c

static void ILI9341_FillColor(uint32_t ulAmout_Point, uint16_t usColor){

uint32_t i = 0;

ILI9341_WriteCmd(CMD_SetPixel);

for(i = 0; i < ulAmout_Point; i++){

ILI9341_WriteData(usColor);

}

}

void LCD_SetColors(uint16_t TextColor, uint16_t BackColor){

g_LCD_TextColor = TextColor;

g_LCD_BackColor = BackColor;

}

void LCD_GetColors(uint16_t *TextColor, uint16_t *BackColor){

*TextColor = g_LCD_TextColor;

*BackColor = g_LCD_BackColor;

}

void LCD_SetTextColor(uint16_t Color){ g_LCD_TextColor = Color; }

void LCD_SetBackColor(uint16_t Color){ g_LCD_BackColor = Color; }

void LCD_Clear(uint16_t usX, uint16_t usY, uint16_t usWidth, uint16_t usHeight){

ILI9341_OpenWindow(usX, usY, usWidth, usHeight);

ILI9341_FillColor(usWidth * usHeight, g_LCD_BackColor);

}

void LCD_SetPixel(uint16_t uX, uint16_t uY){

if((uX < g_LCD_LenX) && (uY < g_LCD_LenY)){

ILI9341_OpenWindow(uX, uY, 1, 1);

ILI9341_FillColor(1, g_LCD_TextColor);

}

}

【代码7-4-3】BSP/lcd_ili9341.c — 字模显示函数

📄 来源文件: lcd_ili9341.c

void LCD_DispMask(uint16_t uX, uint16_t uY, uint16_t uWidth,

int16_t uHeight, const uint8_t *pMask)

{

uint16_t i, j = 0;

uint16_t uFontLenght = 0;

ILI9341_OpenWindow(uX, uY, uWidth, uHeight);

ILI9341_WriteCmd(CMD_SetPixel);

uFontLenght = uWidth * uHeight / 8;

for(i=0; i < uFontLenght; i++){

for(j=0; j < 8; j++){

if(pMask[i] & (0x80>>j)){

ILI9341_WriteData(g_LCD_TextColor);

} else {

ILI9341_WriteData(g_LCD_BackColor);

}

}

}

}

【代码7-4-4】BSP/lcd_ili9341.c — 英文字符串显示函数

📄 来源文件: lcd_ili9341.c

void LCD_DispStringEN(uint16_t uX, uint16_t uY, uint8_t uDir, char *pStr){

uint8_t *pMask = NULL;

uint16_t uCharWidth  = LCD_GetFontEN()->Width;

uint16_t uCharHeight = LCD_GetFontEN()->Height;

while(*pStr != '\0'){

pMask = LCD_GetMaskEN(*pStr);

pStr++;

if(g_LCD_LenX - uX < uCharWidth){

uX = 0;

uY += uCharHeight;

}

if(g_LCD_LenY - uY < uCharHeight){

uY = 0;

}

LCD_DispMask(uX, uY, uCharWidth, uCharHeight, pMask);

if(uDir == 0){

uX += uCharWidth;

} else {

uX += uCharHeight;

}

}

}

【代码7-4-1】BSP/lcd_fonts.h — LCD字体文件接口头文件

📄 来源文件: lcd_fonts.h

#ifndef __LCD_FONT_H__

#define __LCD_FONT_H__

#include "stm32f4xx.h"

typedef struct _tFont{

const uint8_t *table;

uint16_t Width;

uint16_t Height;

}FONT;

extern FONT ASCII_8x16;

#define LINE_EN(x) ((x)*(((FONT *)LCD_GetFontEN())->Height))

FONT *LCD_GetFontEN(void);

void  LCD_SetFontEN(FONT *fonts);

uint8_t *LCD_GetMaskEN(char cChar);

#endif

【代码7-4-2】BSP/lcd_fonts.c — LCD字体实现（ASCII 8x16字模数据+接口函数）

📄 来源文件: lcd_fonts.c

#include <stddef.h>

#include "lcd_fonts.h"

const uint8_t ASCII8x16_Table[] = {

// ASCII 8x16 字模数据（95个可打印字符，每字符16字节）

// 示例：'!' 字符

0x00,0x00,0x00,0x18,0x18,0x18,0x18,0x18,0x18,0x08,0x00,0x08,0x18,0x00,0x00,0x00,

// ... (完整字模表共 95x16 = 1520 字节)

};

FONT ASCII_8x16 = {

ASCII8x16_Table,

8,  /* 字宽 */

16, /* 字高 */

};

static FONT *g_LCD_FontEN = &ASCII_8x16;

void LCD_SetFontEN(FONT *fonts){ g_LCD_FontEN = fonts; }

FONT *LCD_GetFontEN(void){ return g_LCD_FontEN; }

uint8_t *LCD_GetMaskEN(char cChar){

return (uint8_t*)(g_LCD_FontEN->table +

(cChar-' ')*(g_LCD_FontEN->Width*g_LCD_FontEN->Height)/8);

}

【代码7-5-2】USER/main.c — LCD测试主程序

📄 来源文件: main.c

#include <stdio.h>

#include "stm32f4xx.h"

#include "usart.h"

#include "lcd_ili9341.h"

#include "delay.h"

int main(void){

UART2_Init(115200);

printf("LCD TEST\r\n");

LCD_Init();

LCD_SetFontEN(&ASCII_8x16);

LCD_SetColors(RED, BLACK);

LCD_Clear(0, 0, LCD_GetLenX(), LCD_GetLenY());

LCD_DispStringEN(0, LINE_EN(0), 0, "3.2 inch LCD:");

LCD_DispStringEN(0, LINE_EN(1), 0, "Image resolution:240x320 px");

while(1){ }

}

# 实验九 XPT触摸屏实验

【代码7-1-1】BSP/xpt2046.h — XPT2046触摸屏接口头文件

📄 来源文件: xpt2046.h

#ifndef __XPT2046_H__

#define __XPT2046_H__

#include "stm32f4xx.h"

typedef struct{

int16_t x;

int16_t y;

int16_t pre_x;

int16_t pre_y;

} XPT2046_Coord;

typedef struct{

long double A;

long double B;

long double C;

long double D;

long double E;

long double F;

} XPT2046_Factor;

typedef enum {

XPT2046_STATE_RELEASE  = 0,

XPT2046_STATE_WAITING,

XPT2046_STATE_PRESSED,

}enumTouchState;

void    XPT2046_Init(void);

uint8_t XPT2046_DetectTouch(void);

int8_t XPT2046_GetTouchPoint(XPT2046_Coord *displayPtr);

int8_t XPT2046_CalibrationFactor(XPT2046_Coord *pLCD,

XPT2046_Coord *pTouch, XPT2046_Factor *pFactor);

#endif

【代码7-1-2】BSP/xpt2046.c — XPT2046全局定义与SPI引脚宏

📄 来源文件: xpt2046.c

#include <stdio.h>

#include <string.h>

#include "xpt2046.h"

#include "delay.h"

#define XPT2046_CHANNEL_X     0x90

#define XPT2046_CHANNEL_Y     0xd0

#define TOUCH_PRESSED        1

#define TOUCH_NOT_PRESSED    0

#define XPT2046_PENIRQ_Active      0

#define XPT2046_THRESHOLD_CalDiff  2

#define DURIATION_TIME    2

#define XPT2046_CS_ENABLE()    GPIO_ResetBits(GPIOD, GPIO_Pin_13)

#define XPT2046_CS_DISABLE()   GPIO_SetBits(GPIOD, GPIO_Pin_13)

#define XPT2046_CLK_HIGH()      GPIO_SetBits(GPIOE, GPIO_Pin_0)

#define XPT2046_CLK_LOW()       GPIO_ResetBits(GPIOE, GPIO_Pin_0)

#define XPT2046_MOSI_1()       GPIO_SetBits(GPIOE, GPIO_Pin_2)

#define XPT2046_MOSI_0()       GPIO_ResetBits(GPIOE, GPIO_Pin_2)

#define XPT2046_MISO()          GPIO_ReadInputDataBit(GPIOE, GPIO_Pin_3)

#define XPT2046_ReadIRQ()       GPIO_ReadInputDataBit(GPIOE, GPIO_Pin_4)

int  LCD_SCAN_MODE = 0;

XPT2046_Factor TouchFactor[] = {

-0.006464, -0.073259, 280.358032, 0.074878, 0.002052, -6.545977,

0.086314, 0.001891, -12.836658, -0.003722, -0.065799, 254.715714,

0.002782, 0.061522, -11.595689, 0.083393, 0.005159, -15.650089,

0.089743, -0.000289, -20.612209, -0.001374, 0.064451, -16.054003,

0.000767, -0.068258, 250.891769, -0.085559, -0.000195, 334.747650,

-0.084744, 0.000047, 323.163147, -0.002109, -0.066371, 260.985809,

-0.001848, 0.066984, -12.807136, -0.084858, -0.000805, 333.395386,

-0.085470, -0.000876, 334.023163, -0.003390, 0.064725, -6.211169,

};

【代码7-2-1】BSP/xpt2046.c — XPT2046 GPIO初始化

📄 来源文件: xpt2046.c

void XPT2046_Init(void){

GPIO_InitTypeDef  GPIO_InitStructure;

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOD | RCC_AHB1Periph_GPIOE, ENABLE);

//SPI CLK

GPIO_InitStructure.GPIO_Pin    = GPIO_Pin_0;

GPIO_InitStructure.GPIO_Speed  = GPIO_Speed_25MHz;

GPIO_InitStructure.GPIO_Mode   = GPIO_Mode_OUT;

GPIO_InitStructure.GPIO_OType  = GPIO_OType_PP;

GPIO_InitStructure.GPIO_PuPd   = GPIO_PuPd_NOPULL;

GPIO_Init(GPIOE, &GPIO_InitStructure);

//SPI MOSI

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_2;

GPIO_Init(GPIOE, &GPIO_InitStructure);

//SPI CS

GPIO_InitStructure.GPIO_Pin   = GPIO_Pin_13;

GPIO_Init(GPIOD, &GPIO_InitStructure);

//MISO

GPIO_InitStructure.GPIO_Pin   = GPIO_Pin_3;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_25MHz;

GPIO_InitStructure.GPIO_Mode  = GPIO_Mode_IN;

GPIO_InitStructure.GPIO_PuPd  = GPIO_PuPd_UP;

GPIO_Init(GPIOE, &GPIO_InitStructure);

//IRQ

GPIO_InitStructure.GPIO_Pin   = GPIO_Pin_4;

GPIO_InitStructure.GPIO_Mode  = GPIO_Mode_IN;

GPIO_InitStructure.GPIO_PuPd  = GPIO_PuPd_UP;

GPIO_Init(GPIOE, &GPIO_InitStructure);

XPT2046_CS_ENABLE();

}

【代码7-2-3】BSP/xpt2046.c — SPI命令写入与数据读取（GPIO模拟SPI）

📄 来源文件: xpt2046.c

static void XPT2046_WriteCMD(uint8_t ucCmd){

uint8_t i = 0;

XPT2046_MOSI_0();

XPT2046_CLK_LOW();

for(i = 0; i < 8; i ++){

((ucCmd >> (7 - i)) & 0x01) ? XPT2046_MOSI_1() : XPT2046_MOSI_0();

XPT2046_DelayUS(5);

XPT2046_CLK_HIGH();

XPT2046_DelayUS(5);

XPT2046_CLK_LOW();

}

}

static uint16_t XPT2046_ReadCMD(void){

uint8_t i;

uint16_t usBuf=0, usTemp;

XPT2046_MOSI_0();

XPT2046_CLK_HIGH();

for(i=0; i<12; i++){

XPT2046_CLK_LOW();

usTemp = XPT2046_MISO();

usBuf |= usTemp << (11 - i);

XPT2046_CLK_HIGH();

}

return usBuf;

}

【代码7-2-4】BSP/xpt2046.c — ADC通道读取（X/Y轴采集）

📄 来源文件: xpt2046.c

static uint16_t XPT2046_ReadAdc(uint8_t ucChannel){

XPT2046_WriteCMD(ucChannel);

return XPT2046_ReadCMD();

}

static void XPT2046_ReadAdcXY(uint16_t *sX_Ad, uint16_t *sY_Ad){

*sX_Ad = XPT2046_ReadAdc(XPT2046_CHANNEL_X);

XPT2046_DelayUS(1);

*sY_Ad = XPT2046_ReadAdc(XPT2046_CHANNEL_Y);

}

【代码7-3-1】BSP/xpt2046.c — 数值滤波（10次采样去极值取平均）

📄 来源文件: xpt2046.c

static int8_t XPT2046_FilterXY(XPT2046_Coord *pCoord){

uint8_t ucCount = 0, i = 0;

uint16_t value = 0;

uint16_t tempAD_X, tempAD_Y = 0;

uint16_t sBufferArray[2][10] = { {0},{0} };

uint32_t lX_Min, lX_Max, lY_Min, lY_Max = 0;

do{

XPT2046_ReadAdcXY(&tempAD_X, &tempAD_Y);

sBufferArray[0][ucCount] = tempAD_X;

sBufferArray[1][ucCount] = tempAD_Y;

ucCount++;

value = XPT2046_ReadIRQ();

}while((value == XPT2046_PENIRQ_Active) && (ucCount < 10));

if(ucCount < 10){ return -1; }

lX_Max = lX_Min = sBufferArray[0][0];

lY_Max = lY_Min = sBufferArray[1][0];

tempAD_X = sBufferArray[0][0];

tempAD_Y = sBufferArray[1][0];

for(i = 1; i < 10; i++){

if(sBufferArray[0][i] < lX_Min) lX_Min = sBufferArray[0][i];

else if(sBufferArray[0][i] > lX_Max) lX_Max = sBufferArray[0][i];

tempAD_X += sBufferArray[0][i];

}

for(i = 1; i < 10; i++){

if(sBufferArray[1][i] < lY_Min) lY_Min = sBufferArray[1][i];

else if(sBufferArray[1][i] > lY_Max) lY_Max = sBufferArray[1][i];

tempAD_Y += sBufferArray[1][i];

}

pCoord->x = (tempAD_X - lX_Min - lX_Max) >> 3;

pCoord->y = (tempAD_Y - lY_Min - lY_Max) >> 3;

return 0;

}

【代码7-3-3】BSP/xpt2046.c — ADC到LCD坐标转换（三点校准公式）

📄 来源文件: xpt2046.c

int8_t XPT2046_GetTouchPoint(XPT2046_Coord *pLCD){

XPT2046_Coord adcCoord;

if(XPT2046_FilterXY(&adcCoord) < 0){ return -1; }

// XL = AX + BY + C

pLCD->x = TouchFactor[LCD_SCAN_MODE].A * adcCoord.x +

TouchFactor[LCD_SCAN_MODE].B * adcCoord.y +

TouchFactor[LCD_SCAN_MODE].C;

// YL = DX + EY + F

pLCD->y = TouchFactor[LCD_SCAN_MODE].D * adcCoord.x +

TouchFactor[LCD_SCAN_MODE].E * adcCoord.y +

TouchFactor[LCD_SCAN_MODE].F;

return 0;

}

__weak void XPT2046_TouchDown(XPT2046_Coord *touch){

printf("TouchDown,x:%d,y:%d\r\n", touch->x, touch->y);

}

__weak void XPT2046_TouchUp(XPT2046_Coord *touch){

printf("TouchUp,x:%d,y:%d\r\n", touch->x, touch->y);

}

【代码7-3-4】BSP/xpt2046.c — 触摸事件检测状态机（含去抖）

📄 来源文件: xpt2046.c

uint8_t XPT2046_DetectTouch(void){

uint8_t detectResult = TOUCH_NOT_PRESSED;

uint16_t value = 0;

static uint32_t i = 0;

static enumTouchState touch_state = XPT2046_STATE_RELEASE;

static XPT2046_Coord cinfo = {-1,-1,-1,-1};

value = XPT2046_ReadIRQ();

switch(touch_state){

case XPT2046_STATE_RELEASE:

if(value == XPT2046_PENIRQ_Active){

touch_state  = XPT2046_STATE_WAITING;

detectResult = TOUCH_NOT_PRESSED;

} else {

touch_state  = XPT2046_STATE_RELEASE;

detectResult = TOUCH_NOT_PRESSED;

}

break;

case XPT2046_STATE_WAITING:

if(value == XPT2046_PENIRQ_Active){

i++;

if(i > DURIATION_TIME){

i = 0;

touch_state  = XPT2046_STATE_PRESSED;

detectResult = TOUCH_PRESSED;

} else {

touch_state  = XPT2046_STATE_WAITING;

detectResult = TOUCH_NOT_PRESSED;

}

} else {

i = 0;

touch_state  = XPT2046_STATE_RELEASE;

detectResult = TOUCH_NOT_PRESSED;

}

break;

case XPT2046_STATE_PRESSED:

if(value == XPT2046_PENIRQ_Active){

touch_state  = XPT2046_STATE_PRESSED;

detectResult = TOUCH_PRESSED;

} else {

touch_state  = XPT2046_STATE_RELEASE;

detectResult = TOUCH_NOT_PRESSED;

}

break;

default:

touch_state  = XPT2046_STATE_RELEASE;

detectResult = TOUCH_NOT_PRESSED;

break;

}

if(detectResult == TOUCH_PRESSED){

if(XPT2046_GetTouchPoint(&cinfo) < 0){ return detectResult; }

XPT2046_TouchDown(&cinfo);

cinfo.pre_x = cinfo.x;

cinfo.pre_y = cinfo.y;

} else {

if(cinfo.pre_x == -1 && cinfo.pre_x == -1){ return detectResult; }

XPT2046_TouchUp(&cinfo);

cinfo.x = -1; cinfo.y = -1;

cinfo.pre_x = -1; cinfo.pre_y = -1;

}

return detectResult;

}

【代码7-4-2】USER/main.c — 触摸屏测试主程序

📄 来源文件: main.c

#include <stdio.h>

#include "stm32f4xx.h"

#include "xpt2046.h"

#include "usart.h"

#include "delay.h"

int main(void){

UART2_Init(115200);

XPT2046_Init();

printf("XPT2046 TEST\r\n");

while(1){

XPT2046_DetectTouch();

delay_ms(50);

}

}