ARM外围传感器实验 —— 全部代码合集

（实验1~6，共6个实验）

实验1 DHT11温湿度传感器实验

【实验1 DHT11】 文件：实验9_DHT11驱动_dht11.h

#ifndef __DHT11_H__ #define __DHT11_H__ #include "stm32f4xx.h" typedef struct { uint8_t humi_int; uint8_t humi_deci; uint8_t temp_int; uint8_t temp_deci; uint8_t check_sum; } DHT11_Data; void DHT11_Init(void); int8_t DHT11_ReadData(DHT11_Data *pData); #endif

【实验1 DHT11】 文件：实验9_DHT11驱动_dht11.c

#include <stddef.h> #include <stdio.h> #include "dht11.h" #include "delay.h" #define DHT11_PORT GPIOE #define DHT11_PIN GPIO_Pin_6 static void DHT11_Mode_IN(void); static void DHT11_Mode_OUT(void); #define DHT11_DELAY_US(us) delay_us(us) #define DHT11_DELAY_MS(ms) delay_ms(ms) static void DHT11_Mode_IN(void) { GPIO_InitTypeDef GPIO_InitStruct; GPIO_InitStruct.GPIO_Pin = DHT11_PIN; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_IN; GPIO_InitStruct.GPIO_OType = GPIO_OType_PP; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_NOPULL; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_25MHz; GPIO_Init(DHT11_PORT, &GPIO_InitStruct); } static void DHT11_Mode_OUT(void) { GPIO_InitTypeDef GPIO_InitStruct; GPIO_InitStruct.GPIO_Pin = DHT11_PIN; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_OUT; GPIO_InitStruct.GPIO_OType = GPIO_OType_PP; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_NOPULL; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_25MHz; GPIO_Init(DHT11_PORT, &GPIO_InitStruct); } void DHT11_Init(void) { RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOE, ENABLE); DHT11_Mode_OUT(); GPIO_SetBits(DHT11_PORT, DHT11_PIN); } int8_t DHT11_WaitFor(BitAction bitValue, uint32_t max_us) { uint8_t try_num = 0; while((GPIO_ReadInputDataBit(DHT11_PORT, DHT11_PIN) != bitValue) && try_num < max_us) { try_num++; DHT11_DELAY_US(1); } if(try_num >= max_us) return -1; return 0; } int8_t DHT11_ReadBit(void) { if(DHT11_WaitFor(Bit_RESET, 100) < 0) return -1; if(DHT11_WaitFor(Bit_SET, 100) < 0) return -1; DHT11_DELAY_US(40); if(GPIO_ReadInputDataBit(DHT11_PORT, DHT11_PIN) == Bit_SET) return 1; else return 0; } static uint8_t DHT11_ReadByte(void) { uint8_t byte = 0; uint8_t i; for(i = 0; i < 8; i++) { byte <<= 1; byte ¦= DHT11_ReadBit(); } return byte; } int8_t DHT11_ReadData(DHT11_Data *pData) { if(pData == NULL) return -1; DHT11_Mode_OUT(); GPIO_SetBits(DHT11_PORT, DHT11_PIN); GPIO_ResetBits(DHT11_PORT, DHT11_PIN); DHT11_DELAY_MS(18); GPIO_SetBits(DHT11_PORT, DHT11_PIN); DHT11_DELAY_US(30); DHT11_Mode_IN(); if(DHT11_WaitFor(Bit_RESET, 200) < 0) return -1; if(DHT11_WaitFor(Bit_SET, 100) < 0) return -1; DHT11_DELAY_US(80); pData->humi_int = DHT11_ReadByte(); pData->humi_deci = DHT11_ReadByte(); pData->temp_int = DHT11_ReadByte(); pData->temp_deci = DHT11_ReadByte(); pData->check_sum = DHT11_ReadByte(); if(pData->check_sum != pData->humi_int + pData->humi_deci + pData->temp_int + pData->temp_deci) { return -1; } return 0; }

【实验1 DHT11】 文件：实验9_DHT11驱动_usart.h

#ifndef __USART_H__ #define __USART_H__ #include "stm32f4xx.h" void UART2_Init(u32 bound); #endif

【实验1 DHT11】 文件：实验9_DHT11驱动_usart.c

#include <stdio.h> #include "usart.h" void UART2_Init(u32 bound) { GPIO_InitTypeDef GPIO_InitStructure; USART_InitTypeDef USART_InitStructure; NVIC_InitTypeDef NVIC_InitStructure; RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA, ENABLE); RCC_APB1PeriphClockCmd(RCC_APB1Periph_USART2, ENABLE); GPIO_PinAFConfig(GPIOA, GPIO_PinSource2, GPIO_AF_USART2); GPIO_PinAFConfig(GPIOA, GPIO_PinSource3, GPIO_AF_USART2); GPIO_InitStructure.GPIO_Pin = GPIO_Pin_2 ¦ GPIO_Pin_3; GPIO_InitStructure.GPIO_Mode = GPIO_Mode_AF; GPIO_InitStructure.GPIO_Speed = GPIO_Speed_50MHz; GPIO_InitStructure.GPIO_OType = GPIO_OType_PP; GPIO_InitStructure.GPIO_PuPd = GPIO_PuPd_UP; GPIO_Init(GPIOA, &GPIO_InitStructure); USART_InitStructure.USART_BaudRate = bound; USART_InitStructure.USART_WordLength = USART_WordLength_8b; USART_InitStructure.USART_StopBits = USART_StopBits_1; USART_InitStructure.USART_Parity = USART_Parity_No; USART_InitStructure.USART_HardwareFlowControl = USART_HardwareFlowControl_None; USART_InitStructure.USART_Mode = USART_Mode_Rx ¦ USART_Mode_Tx; USART_Init(USART2, &USART_InitStructure); NVIC_InitStructure.NVIC_IRQChannel = USART2_IRQn; NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 3; NVIC_InitStructure.NVIC_IRQChannelSubPriority = 3; NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE; NVIC_Init(&NVIC_InitStructure); USART_ITConfig(USART2, USART_IT_RXNE, ENABLE); USART_Cmd(USART2, ENABLE); } void UART_SendByte(USART_TypeDef *pUSARTx, uint8_t ch) { USART_SendData(pUSARTx, ch); while (USART_GetFlagStatus(pUSARTx, USART_FLAG_TXE) == RESET); } void UART_SendString(USART_TypeDef *pUSARTx, char *str) { unsigned int k = 0; do { UART_SendByte(pUSARTx, *(str + k)); k++; } while(*(str + k) != '\0'); while(USART_GetFlagStatus(pUSARTx, USART_FLAG_TC) == RESET); } void UART_SendHalfWord(USART_TypeDef *pUSARTx, uint16_t ch) { uint8_t temp_h, temp_l; temp_h = (ch & 0XFF00) >> 8; temp_l = ch & 0XFF; USART_SendData(pUSARTx, temp_h); while (USART_GetFlagStatus(pUSARTx, USART_FLAG_TXE) == RESET); USART_SendData(pUSARTx, temp_l); while (USART_GetFlagStatus(pUSARTx, USART_FLAG_TXE) == RESET); } int fputc(int ch, FILE *f) { USART_SendData(USART2, (uint8_t)ch); while (USART_GetFlagStatus(USART2, USART_FLAG_TXE) == RESET); return (ch); } int fgetc(FILE *f) { while (USART_GetFlagStatus(USART2, USART_FLAG_RXNE) == RESET); return (int)USART_ReceiveData(USART2); } void USART2_IRQHandler(void) { u8 sbuf = 0; if(USART_GetFlagStatus(USART2, USART_FLAG_RXNE)) { sbuf = USART_ReceiveData(USART2); USART_SendData(USART2, sbuf); } }

【实验1 DHT11】 文件：实验9_DHT11温湿度传感器.c (main)

#include <stdio.h> #include "stm32f4xx.h" #include "dht11.h" #include "usart.h" #include "delay.h" int main(void) { DHT11_Data Data; UART2_Init(115200); DHT11_Init(); printf("DHT11 Init OK\r\n"); delay_ms(1000); while(1) { if(DHT11_ReadData(&Data) == 0) { printf("HUMI:%d.%d_TEMP:%d.%d\r\n", Data.humi_int, Data.humi_deci, Data.temp_int, Data.temp_deci); } else { printf("Read DHT11 ERROR!\r\n"); } delay_ms(10000); } }

实验2 MQ2燃气传感器实验（选做）

【实验2 MQ2】 文件：实验10_MQ2驱动_mq.h

#ifndef __MQ_H__ #define __MQ_H__ #include "stm32f4xx.h" void MQ_Init(void); uint16_t MQ_ReadValue(void); #endif

【实验2 MQ2】 文件：实验10_MQ2驱动_mq.c

#include "stm32f4xx.h" #include "mq.h" #include "delay.h" #define MQ_ADC ADC1 void MQ_GPIO_Config(void) { GPIO_InitTypeDef GPIO_InitX; RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOC, ENABLE); GPIO_InitX.GPIO_Pin = GPIO_Pin_2; GPIO_InitX.GPIO_Mode = GPIO_Mode_AIN; GPIO_InitX.GPIO_PuPd = GPIO_PuPd_NOPULL; GPIO_Init(GPIOC, &GPIO_InitX); } void MQ_ADC_Config(void) { ADC_InitTypeDef ADC_InitX; ADC_CommonInitTypeDef ADC_CommonInitX; RCC_APB2PeriphClockCmd(RCC_APB2Periph_ADC1, ENABLE); ADC_CommonInitX.ADC_Mode = ADC_Mode_Independent; ADC_CommonInitX.ADC_Prescaler = ADC_Prescaler_Div4; ADC_CommonInitX.ADC_DMAAccessMode = ADC_DMAAccessMode_Disabled; ADC_CommonInitX.ADC_TwoSamplingDelay = ADC_TwoSamplingDelay_20Cycles; ADC_CommonInit(&ADC_CommonInitX); ADC_StructInit(&ADC_InitX); ADC_InitX.ADC_Resolution = ADC_Resolution_12b; ADC_InitX.ADC_ScanConvMode = DISABLE; ADC_InitX.ADC_ContinuousConvMode = DISABLE; ADC_InitX.ADC_ExternalTrigConvEdge = ADC_ExternalTrigConvEdge_None; ADC_InitX.ADC_ExternalTrigConv = ADC_ExternalTrigConv_T1_CC1; ADC_InitX.ADC_DataAlign = ADC_DataAlign_Right; ADC_InitX.ADC_NbrOfConversion = 1; ADC_Init(MQ_ADC, &ADC_InitX); ADC_RegularChannelConfig(MQ_ADC, ADC_Channel_12, 1, ADC_SampleTime_28Cycles); ADC_Cmd(MQ_ADC, ENABLE); } void MQ_Init(void) { MQ_GPIO_Config(); MQ_ADC_Config(); } uint16_t MQ_ReadValue(void) { uint32_t uTimeout = 100; ADC_SoftwareStartConv(MQ_ADC); while(ADC_GetFlagStatus(MQ_ADC, ADC_FLAG_EOC) != SET) { if(uTimeout--) { return 0; } delay_ms(1); } ADC_ClearFlag(MQ_ADC, ADC_FLAG_EOC); return ADC_GetConversionValue(MQ_ADC); }

【实验2 MQ2】 文件：实验10_MQ2燃气传感器.c (main)

#include <stdio.h> #include "stm32f4xx.h" #include "mq.h" #include "usart.h" #include "delay.h" int main(void) { uint16_t uValue = 0; float fVolValue = 0.0; UART2_Init(115200); MQ_Init(); printf("MQ Init OK\r\n"); while(1) { uValue = MQ_ReadValue(); fVolValue = (float)uValue / 4096 * (float)3.3; printf("AD=%04d, %fV\r\n", uValue, fVolValue); delay_ms(1000); } }

实验3 IR红外发送实验

【实验3 IR发送】 文件：ir_send.h

#ifndef __IR_SEND_H__ #define __IR_SEND_H__ #include "stm32f4xx.h" void IR_Send_Init(void); void PwmEnable(void); void PwmDisable(void); void IR_Sent_Byte(uint8_t data); void IR_Send(uint8_t addr, uint8_t code); #endif

【实验3 IR发送】 文件：ir_send.c

#include "stm32f4xx.h" #include "ir_send.h" #include "delay.h" #define IR_SEND_TIM TIM10 #define IR_SEND_PORT GPIOB #define IR_SEND_PIN GPIO_Pin_8 #define IR_SEND_DELAY_US(us) delay_us(us) // PB8 -> TIM10_CH1, 38KHz PWM载波 static void IR_Send_GPIO_Init(void) { GPIO_InitTypeDef GPIO_InitStruct; RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOB, ENABLE); GPIO_InitStruct.GPIO_Pin = IR_SEND_PIN; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_AF; GPIO_InitStruct.GPIO_OType = GPIO_OType_PP; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_100MHz; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_UP; GPIO_Init(IR_SEND_PORT, &GPIO_InitStruct); GPIO_PinAFConfig(IR_SEND_PORT, GPIO_PinSource8, GPIO_AF_TIM10); } // TIM10: 168MHz / 1 / 4421 = 38KHz static void IR_Send_PWM_Init(void) { TIM_TimeBaseInitTypeDef TIM_TimeBaseStruct; TIM_OCInitTypeDef TIM_OCInitStruct; RCC_APB2PeriphClockCmd(RCC_APB2Periph_TIM10, ENABLE); TIM_TimeBaseStruct.TIM_Period = 4421 - 1; TIM_TimeBaseStruct.TIM_Prescaler = 1 - 1; TIM_TimeBaseStruct.TIM_ClockDivision = TIM_CKD_DIV1; TIM_TimeBaseStruct.TIM_CounterMode = TIM_CounterMode_Up; TIM_TimeBaseInit(IR_SEND_TIM, &TIM_TimeBaseStruct); TIM_OCInitStruct.TIM_OCMode = TIM_OCMode_PWM1; TIM_OCInitStruct.TIM_OutputState = TIM_OutputState_Enable; TIM_OCInitStruct.TIM_Pulse = 2210; // 50% duty TIM_OCInitStruct.TIM_OCPolarity = TIM_OCPolarity_High; TIM_OC1Init(IR_SEND_TIM, &TIM_OCInitStruct); TIM_OC1PreloadConfig(IR_SEND_TIM, TIM_OCPreload_Enable); TIM_ARRPreloadConfig(IR_SEND_TIM, ENABLE); } void IR_Send_Init(void) { IR_Send_GPIO_Init(); IR_Send_PWM_Init(); } void PwmEnable(void) { TIM_ForcedOC1Config(IR_SEND_TIM, TIM_OCMode_PWM1); TIM_Cmd(IR_SEND_TIM, ENABLE); } void PwmDisable(void) { TIM_ForcedOC1Config(IR_SEND_TIM, TIM_ForcedAction_InActive); TIM_Cmd(IR_SEND_TIM, DISABLE); } void IR_Sent_Byte(uint8_t data) { uint8_t i; for(i = 0; i < 8; i++) { PwmEnable(); IR_SEND_DELAY_US(560); PwmDisable(); if(data & 0x01) IR_SEND_DELAY_US(1680); // bit 1 else IR_SEND_DELAY_US(560); // bit 0 data >>= 1; } } void IR_Send(uint8_t addr, uint8_t code) { // 引导头: 9ms载波 + 4.5ms间隔 PwmEnable(); IR_SEND_DELAY_US(9000); PwmDisable(); IR_SEND_DELAY_US(4500); // 地址码 + 地址反码 IR_Sent_Byte(addr); IR_Sent_Byte(~addr); // 键码 + 键码反码 IR_Sent_Byte(code); IR_Sent_Byte(~code); // 停止位 PwmEnable(); IR_SEND_DELAY_US(560); PwmDisable(); }

【实验3 IR发送】 文件：main.c (测试代码)

#include "stm32f4xx.h" #include "ir_send.h" #include "ir_recv.h" #include "usart.h" #include "delay.h" #include <stdio.h> volatile uint32_t uMSCount = 0; void SysTick_Handler(void) { uMSCount++; } int main(void) { SysTick_Config(SystemCoreClock / 1000); UART2_Init(115200); IR_Recv_Init(); IR_Send_Init(); printf("IR Send & Recv Test Start\r\n"); while(1) { IR_Recv(); // 实时检测接收 if(uMSCount >= 10000) // 每10秒发一次 { uMSCount = 0; printf("IRSend: addr:1, code:123\r\n"); IR_Send(1, 123); } } }

实验4 IR红外接收实验

【实验4 IR接收】 文件：ir_recv.h

#ifndef __IR_RECV_H__ #define __IR_RECV_H__ #include "stm32f4xx.h" void IR_Recv_Init(void); void IR_Recv(void); #endif

【实验4 IR接收】 文件：ir_recv.c

#include "ir_recv.h" #include "usart.h" #include <stdio.h> #define IR_RECV_TIM TIM9 #define IR_RECV_PORT GPIOE #define IR_RECV_PIN GPIO_Pin_5 // 红外接收状态 #define IR_STATE_IDLE 0 #define IR_STATE_START 1 #define IR_STATE_DATA 2 static volatile uint8_t ir_state = IR_STATE_IDLE; static volatile uint32_t ir_data = 0; static volatile uint8_t ir_bit_count = 0; static volatile uint32_t last_capture = 0; static volatile uint8_t ir_recv_ok = 0; static volatile uint32_t ir_recv_value = 0; void IR_TIM_Init(void) { GPIO_InitTypeDef GPIO_InitStruct; TIM_TimeBaseInitTypeDef TIM_TimeBaseStruct; TIM_ICInitTypeDef TIM_ICInitStruct; NVIC_InitTypeDef NVIC_InitStruct; RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOE, ENABLE); RCC_APB2PeriphClockCmd(RCC_APB2Periph_TIM9, ENABLE); // PE5 -> TIM9_CH1 GPIO_InitStruct.GPIO_Pin = IR_RECV_PIN; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_AF; GPIO_InitStruct.GPIO_OType = GPIO_OType_PP; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_100MHz; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_UP; GPIO_Init(IR_RECV_PORT, &GPIO_InitStruct); GPIO_PinAFConfig(IR_RECV_PORT, GPIO_PinSource5, GPIO_AF_TIM9); // 计数频率 1MHz, 周期 10ms TIM_TimeBaseStruct.TIM_Period = 10000; TIM_TimeBaseStruct.TIM_Prescaler = 167; // 168MHz / (167+1) = 1MHz TIM_TimeBaseStruct.TIM_ClockDivision = TIM_CKD_DIV1; TIM_TimeBaseStruct.TIM_CounterMode = TIM_CounterMode_Up; TIM_TimeBaseInit(IR_RECV_TIM, &TIM_TimeBaseStruct); // 输入捕获: 上升沿, 不分频, 8周期滤波 TIM_ICInitStruct.TIM_Channel = TIM_Channel_1; TIM_ICInitStruct.TIM_ICPolarity = TIM_ICPolarity_Rising; TIM_ICInitStruct.TIM_ICSelection = TIM_ICSelection_DirectTI; TIM_ICInitStruct.TIM_ICPrescaler = TIM_ICPSC_DIV1; TIM_ICInitStruct.TIM_ICFilter = 8; TIM_ICInit(IR_RECV_TIM, &TIM_ICInitStruct); // 使能捕获中断和更新中断 TIM_ITConfig(IR_RECV_TIM, TIM_IT_CC1 ¦ TIM_IT_Update, ENABLE); NVIC_InitStruct.NVIC_IRQChannel = TIM1_BRK_TIM9_IRQn; NVIC_InitStruct.NVIC_IRQChannelPreemptionPriority = 0; NVIC_InitStruct.NVIC_IRQChannelSubPriority = 1; NVIC_InitStruct.NVIC_IRQChannelCmd = ENABLE; NVIC_Init(&NVIC_InitStruct); TIM_Cmd(IR_RECV_TIM, ENABLE); } void IR_Recv_Init(void) { IR_TIM_Init(); } void TIM1_BRK_TIM9_IRQHandler(void) { uint32_t capture_val, pulse_width; if(TIM_GetITStatus(IR_RECV_TIM, TIM_IT_CC1) == SET) { capture_val = TIM_GetCapture1(IR_RECV_TIM); if(ir_state == IR_STATE_IDLE) { ir_state = IR_STATE_START; ir_bit_count = 0; ir_data = 0; } else { // 计算脉冲宽度 (单位: us) if(capture_val > last_capture) pulse_width = capture_val - last_capture; else pulse_width = (10000 - last_capture) + capture_val; if(ir_state == IR_STATE_START) { // 引导码检测: 4.5ms = 4500us if(pulse_width > 4000 && pulse_width < 5000) { ir_state = IR_STATE_DATA; ir_bit_count = 0; ir_data = 0; } else { ir_state = IR_STATE_IDLE; } } else if(ir_state == IR_STATE_DATA) { // 数据位: bit0=560us间隔, bit1=1680us间隔 if(pulse_width > 1000 && pulse_width < 2500) { ir_data ¦= (1UL << ir_bit_count); // bit 1 } ir_bit_count++; if(ir_bit_count >= 32) { ir_recv_value = ir_data; ir_recv_ok = 1; ir_state = IR_STATE_IDLE; } } } last_capture = capture_val; TIM_ClearITPendingBit(IR_RECV_TIM, TIM_IT_CC1); } if(TIM_GetITStatus(IR_RECV_TIM, TIM_IT_Update) == SET) { // 超时复位 ir_state = IR_STATE_IDLE; TIM_ClearITPendingBit(IR_RECV_TIM, TIM_IT_Update); } } void IR_Recv(void) { if(ir_recv_ok) { ir_recv_ok = 0; uint8_t addr = (ir_recv_value >> 0) & 0xFF; uint8_t addr_inv = (ir_recv_value >> 8) & 0xFF; uint8_t code = (ir_recv_value >> 16) & 0xFF; uint8_t code_inv = (ir_recv_value >> 24) & 0xFF; // 校验 if((addr == (uint8_t)(~addr_inv)) && (code == (uint8_t)(~code_inv))) { printf("IR Recv: addr=%d, code=%d\r\n", addr, code); } } }

【实验4 IR接收】 文件：main.c (测试代码)

#include "stm32f4xx.h" #include "ir_recv.h" #include "usart.h" #include <stdio.h> int main(void) { UART2_Init(115200); IR_Recv_Init(); printf("IR Receive Test Start\r\n"); while(1) { IR_Recv(); } }

实验5 I2C与E2PROM实验

【实验5 I2C/E2PROM】 文件：e2prom_at24c02.h

#ifndef __E2PROM_AT24C02_H__ #define __E2PROM_AT24C02_H__ #include "stm32f4xx.h" #define AT24C02_ADDR 0xA0 void I2C_EE_Init(void); void AT24C02_ByteWrite(uint8_t addr, uint8_t data); uint8_t AT24C02_ByteRead(uint8_t addr); void AT24C02_BufferWrite(uint8_t *pBuffer, uint8_t Addr, uint16_t Number); void AT24C02_BufferRead(uint8_t *pBuffer, uint8_t Addr, uint16_t Number); #endif

【实验5 I2C/E2PROM】 文件：e2prom_at24c02.c

#include "e2prom_at24c02.h" #include "delay.h" // I2C1: PB6=SCL, PB7=SDA void I2C_EE_Init(void) { GPIO_InitTypeDef GPIO_InitStruct; I2C_InitTypeDef I2C_InitStruct; RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOB, ENABLE); RCC_APB1PeriphClockCmd(RCC_APB1Periph_I2C1, ENABLE); GPIO_InitStruct.GPIO_Pin = GPIO_Pin_6 ¦ GPIO_Pin_7; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_AF; GPIO_InitStruct.GPIO_OType = GPIO_OType_OD; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_50MHz; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_UP; GPIO_Init(GPIOB, &GPIO_InitStruct); GPIO_PinAFConfig(GPIOB, GPIO_PinSource6, GPIO_AF_I2C1); GPIO_PinAFConfig(GPIOB, GPIO_PinSource7, GPIO_AF_I2C1); I2C_InitStruct.I2C_Mode = I2C_Mode_I2C; I2C_InitStruct.I2C_DutyCycle = I2C_DutyCycle_2; I2C_InitStruct.I2C_OwnAddress1 = 0x00; I2C_InitStruct.I2C_Ack = I2C_Ack_Enable; I2C_InitStruct.I2C_AcknowledgedAddress = I2C_AcknowledgedAddress_7bit; I2C_InitStruct.I2C_ClockSpeed = 100000; // 100KHz I2C_Init(I2C1, &I2C_InitStruct); I2C_Cmd(I2C1, ENABLE); } void AT24C02_ByteWrite(uint8_t addr, uint8_t data) { while(I2C_GetFlagStatus(I2C1, I2C_FLAG_BUSY)); I2C_GenerateSTART(I2C1, ENABLE); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_MODE_SELECT)); I2C_Send7bitAddress(I2C1, AT24C02_ADDR, I2C_Direction_Transmitter); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_TRANSMITTER_MODE_SELECTED)); I2C_SendData(I2C1, addr); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_BYTE_TRANSMITTED)); I2C_SendData(I2C1, data); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_BYTE_TRANSMITTED)); I2C_GenerateSTOP(I2C1, ENABLE); delay_ms(5); // E2PROM写周期 } uint8_t AT24C02_ByteRead(uint8_t addr) { uint8_t data; while(I2C_GetFlagStatus(I2C1, I2C_FLAG_BUSY)); I2C_GenerateSTART(I2C1, ENABLE); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_MODE_SELECT)); I2C_Send7bitAddress(I2C1, AT24C02_ADDR, I2C_Direction_Transmitter); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_TRANSMITTER_MODE_SELECTED)); I2C_SendData(I2C1, addr); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_BYTE_TRANSMITTED)); I2C_GenerateSTART(I2C1, ENABLE); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_MODE_SELECT)); I2C_Send7bitAddress(I2C1, AT24C02_ADDR, I2C_Direction_Receiver); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_RECEIVER_MODE_SELECTED)); I2C_AcknowledgeConfig(I2C1, DISABLE); while(!I2C_CheckEvent(I2C1, I2C_EVENT_MASTER_BYTE_RECEIVED)); data = I2C_ReceiveData(I2C1); I2C_GenerateSTOP(I2C1, ENABLE); I2C_AcknowledgeConfig(I2C1, ENABLE); return data; } void AT24C02_BufferWrite(uint8_t *pBuffer, uint8_t Addr, uint16_t Number) { while(Number--) { AT24C02_ByteWrite(Addr, *pBuffer); Addr++; pBuffer++; } } void AT24C02_BufferRead(uint8_t *pBuffer, uint8_t Addr, uint16_t Number) { while(Number--) { *pBuffer = AT24C02_ByteRead(Addr); Addr++; pBuffer++; } }

【实验5 I2C/E2PROM】 文件：main.c (测试代码)

#include "stm32f4xx.h" #include "e2prom_at24c02.h" #include "usart.h" #include <stdio.h> int main(void) { uint8_t i; uint8_t write_buf[256]; uint8_t read_buf[256]; uint8_t error_flag = 0; UART2_Init(115200); I2C_EE_Init(); printf("AT24C02 Test Start\r\n"); // 生成0~255测试数据 for(i = 0; i < 256; i++) write_buf[i] = i; // 写入 AT24C02_BufferWrite(write_buf, 0, 256); printf("Write Complete\r\n"); // 读出并比较 AT24C02_BufferRead(read_buf, 0, 256); for(i = 0; i < 256; i++) { if(read_buf[i] != write_buf[i]) { error_flag = 1; break; } } if(error_flag) printf("TEST FAIL at addr %d\r\n", i); else printf("TEST OK\r\n"); while(1); }

实验6 SPI与FLASH实验

【实验6 SPI/FLASH】 文件：flash_w25qxx.h

#ifndef __FLASH_W25QXX_H__ #define __FLASH_W25QXX_H__ #include "stm32f4xx.h" #define W25QXX_FLASH_SIZE (8 * 1024 * 1024) // W25Q64 = 8MB void SPI_Flash_Init(void); uint32_t SPI_Flash_ReadID(void); void SPI_Flash_SectorErase(uint32_t SectorAddr); void SPI_Flash_BufferWrite(uint8_t *pBuffer, uint32_t WriteAddr, uint16_t NumByteToWrite); void SPI_Flash_BufferRead(uint8_t *pBuffer, uint32_t ReadAddr, uint16_t NumByteToRead); #endif

【实验6 SPI/FLASH】 文件：flash_w25qxx.c

#include "flash_w25qxx.h" #include "delay.h" // SPI1: PA5=SCK, PA6=MISO, PA7=MOSI, PA4=NSS #define FLASH_SPI SPI1 #define FLASH_CS_PORT GPIOA #define FLASH_CS_PIN GPIO_Pin_4 #define FLASH_CS_LOW() GPIO_ResetBits(FLASH_CS_PORT, FLASH_CS_PIN) #define FLASH_CS_HIGH() GPIO_SetBits(FLASH_CS_PORT, FLASH_CS_PIN) // W25QXX 指令 #define CMD_WRITE_ENABLE 0x06 #define CMD_SECTOR_ERASE 0x20 #define CMD_READ_DATA 0x03 #define CMD_PAGE_PROGRAM 0x02 #define CMD_READ_ID 0x9F #define CMD_READ_STATUS 0x05 static uint8_t SPI_Flash_SendByte(uint8_t byte) { while(SPI_I2S_GetFlagStatus(FLASH_SPI, SPI_I2S_FLAG_TXE) == RESET); SPI_I2S_SendData(FLASH_SPI, byte); while(SPI_I2S_GetFlagStatus(FLASH_SPI, SPI_I2S_FLAG_RXNE) == RESET); return SPI_I2S_ReceiveData(FLASH_SPI); } static void SPI_Flash_WaitBusy(void) { FLASH_CS_LOW(); SPI_Flash_SendByte(CMD_READ_STATUS); while(SPI_Flash_SendByte(0xFF) & 0x01); FLASH_CS_HIGH(); } static void SPI_Flash_WriteEnable(void) { FLASH_CS_LOW(); SPI_Flash_SendByte(CMD_WRITE_ENABLE); FLASH_CS_HIGH(); } void SPI_Flash_Init(void) { GPIO_InitTypeDef GPIO_InitStruct; SPI_InitTypeDef SPI_InitStruct; RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA, ENABLE); RCC_APB2PeriphClockCmd(RCC_APB2Periph_SPI1, ENABLE); // PA5=SCK, PA6=MISO, PA7=MOSI GPIO_InitStruct.GPIO_Pin = GPIO_Pin_5 ¦ GPIO_Pin_6 ¦ GPIO_Pin_7; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_AF; GPIO_InitStruct.GPIO_OType = GPIO_OType_PP; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_50MHz; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_UP; GPIO_Init(GPIOA, &GPIO_InitStruct); GPIO_PinAFConfig(GPIOA, GPIO_PinSource5, GPIO_AF_SPI1); GPIO_PinAFConfig(GPIOA, GPIO_PinSource6, GPIO_AF_SPI1); GPIO_PinAFConfig(GPIOA, GPIO_PinSource7, GPIO_AF_SPI1); // PA4=NSS (软件控制) GPIO_InitStruct.GPIO_Pin = FLASH_CS_PIN; GPIO_InitStruct.GPIO_Mode = GPIO_Mode_OUT; GPIO_InitStruct.GPIO_OType = GPIO_OType_PP; GPIO_InitStruct.GPIO_Speed = GPIO_Speed_50MHz; GPIO_InitStruct.GPIO_PuPd = GPIO_PuPd_UP; GPIO_Init(FLASH_CS_PORT, &GPIO_InitStruct); FLASH_CS_HIGH(); SPI_InitStruct.SPI_Direction = SPI_Direction_2Lines_FullDuplex; SPI_InitStruct.SPI_Mode = SPI_Mode_Master; SPI_InitStruct.SPI_DataSize = SPI_DataSize_8b; SPI_InitStruct.SPI_CPOL = SPI_CPOL_High; SPI_InitStruct.SPI_CPHA = SPI_CPHA_2Edge; SPI_InitStruct.SPI_NSS = SPI_NSS_Soft; SPI_InitStruct.SPI_BaudRatePrescaler = SPI_BaudRatePrescaler_4; SPI_InitStruct.SPI_FirstBit = SPI_FirstBit_MSB; SPI_InitStruct.SPI_CRCPolynomial = 7; SPI_Init(FLASH_SPI, &SPI_InitStruct); SPI_Cmd(FLASH_SPI, ENABLE); } uint32_t SPI_Flash_ReadID(void) { uint32_t id = 0; FLASH_CS_LOW(); SPI_Flash_SendByte(CMD_READ_ID); id ¦= SPI_Flash_SendByte(0xFF) << 16; id ¦= SPI_Flash_SendByte(0xFF) << 8; id ¦= SPI_Flash_SendByte(0xFF); FLASH_CS_HIGH(); return id; } void SPI_Flash_SectorErase(uint32_t SectorAddr) { SPI_Flash_WriteEnable(); FLASH_CS_LOW(); SPI_Flash_SendByte(CMD_SECTOR_ERASE); SPI_Flash_SendByte((SectorAddr >> 16) & 0xFF); SPI_Flash_SendByte((SectorAddr >> 8) & 0xFF); SPI_Flash_SendByte(SectorAddr & 0xFF); FLASH_CS_HIGH(); SPI_Flash_WaitBusy(); } void SPI_Flash_BufferRead(uint8_t *pBuffer, uint32_t ReadAddr, uint16_t NumByteToRead) { FLASH_CS_LOW(); SPI_Flash_SendByte(CMD_READ_DATA); SPI_Flash_SendByte((ReadAddr >> 16) & 0xFF); SPI_Flash_SendByte((ReadAddr >> 8) & 0xFF); SPI_Flash_SendByte(ReadAddr & 0xFF); while(NumByteToRead--) { *pBuffer = SPI_Flash_SendByte(0xFF); pBuffer++; } FLASH_CS_HIGH(); } void SPI_Flash_BufferWrite(uint8_t *pBuffer, uint32_t WriteAddr, uint16_t NumByteToWrite) { uint16_t i; while(NumByteToWrite > 0) { uint16_t page_remain = 256 - (WriteAddr & 0xFF); uint16_t write_len = (NumByteToWrite > page_remain) ? page_remain : NumByteToWrite; SPI_Flash_WriteEnable(); FLASH_CS_LOW(); SPI_Flash_SendByte(CMD_PAGE_PROGRAM); SPI_Flash_SendByte((WriteAddr >> 16) & 0xFF); SPI_Flash_SendByte((WriteAddr >> 8) & 0xFF); SPI_Flash_SendByte(WriteAddr & 0xFF); for(i = 0; i < write_len; i++) SPI_Flash_SendByte(pBuffer[i]); FLASH_CS_HIGH(); SPI_Flash_WaitBusy(); pBuffer += write_len; WriteAddr += write_len; NumByteToWrite -= write_len; } }

【实验6 SPI/FLASH】 文件：main.c (测试代码)

#include "stm32f4xx.h" #include "flash_w25qxx.h" #include "usart.h" #include <stdio.h> int main(void) { uint16_t i; uint8_t write_buf[256]; uint8_t read_buf[256]; uint32_t flash_id; UART2_Init(115200); SPI_Flash_Init(); printf("W25Q64 SPI Flash Test\r\n"); // 读取ID flash_id = SPI_Flash_ReadID(); printf("Flash ID: 0x%06X\r\n", flash_id); if(flash_id == 0xEF4017) { printf("W25Q64 Detected!\r\n"); // 擦除扇区0 SPI_Flash_SectorErase(0); printf("Sector 0 Erased\r\n"); // 生成测试数据 for(i = 0; i < 256; i++) write_buf[i] = i; // 写入 SPI_Flash_BufferWrite(write_buf, 0, 256); printf("Write Complete\r\n"); // 读出并比较 SPI_Flash_BufferRead(read_buf, 0, 256); for(i = 0; i < 256; i++) { if(read_buf[i] != write_buf[i]) { printf("Error at addr %d: Wr=%02X Rd=%02X\r\n", i, write_buf[i], read_buf[i]); break; } } if(i == 256) printf("TEST OK!\r\n"); } else { printf("Flash ID Error!\r\n"); } while(1); }